REF	2D Adaptable Wall System

Uniclass2015Title	Panel partition systems

Uniclass2015Code Ss_25_12_65_65

General

Colour	Insert RAL Colour and Gloss Level or Anodised Colour Reference

Features 	This offers great flexibility, with simple adaptations enabling panels to be changed from glazed to solid, or vice versa. The system features a deflection head which is compatible with our full range of systems, meaning there is no need to deal with those awkward detail junctions between a dry wall, cross wall and a glazed room frontage any longer.

Finish	Polyester Powder Coated RAL (Insert Reference)

Polished & Chemically brightened (Insert Reference)

Linish & Chemically brightened (Insert Reference)

Etch & Anodise (Insert Reference)

Material 	Aluminium (6063 T6) / glass

Shape	Refer to Architects internal screen schedule / elevation drawing

Size	To suit project requirements - please consult with manufacturer

Warranty	Dealt with on a project specific basis as part of the O&M process

Description

Application	Commercial

Recycled 	17-28% glass / 70% Aluminium

Content

Recyclability	Up to 100% glass / 100% Aluminium

Embodied 	EPD in progress

Carbon

Red Listed 	No substance on the “Candidate List of Substances of Very High Concern for

Materials	authorisation” derived under REACH is present either above the limits for registration with the European Chemicals Agency or in excess of 1 weight–% of the product.

Country Of 	UK

Origin

NBS_Data

System

Manufacturer	Radii

31 Harbour Exchange Square

Canary Wharf London E14 9GE UK

T. 020 7952 4830

E. info@radiipartitioning.com

Performance 	To comply to EN ISO 10140-2 and ISO 717-1: Acoustics vary according to framework/glazing

Acoustic 	options subject to further design development

dB Rw

Panel 		Framed solid/glazed system

Partitioning

Fasteners	To manufacturers specification, type, length and frequency to suit substrates

Joint

Treatment	3mm Shadow-gap to panel joints

Joint Finish	Acoustic Seal

System

Accessories	Consult with RPG

Samples	Consult with RPG

Required

Third Party

Accreditation       ISO 9001-UKAS (Quality)

ISO 14001-UKAS (Environmental)

ISO 45001-UKAS (health & Safety)

Constructionline-Gold

Acclaim (SIPP)

Achilles (Building confidence, SSIP)

FIS Member

Considerate Construction

FORS Bronze

Builders Profile

Investors in People Platinum

Wood In 	N/A

Joinery Standard

Unit Size

Thickness	125mm

Unit Size 	Insert Project Specific Requirement

Partition

Height(nominal)

Unit Size 	Insert Project Specific Requirement

Width

(nominal)

Deflection 	+/- 25mm

Allowance

(maximum)

Head

Deflection	None

Allowance

(maximum)

Floor

Deflection	No lateral deflection

Allowance

(maximum)

Verticals

Panel Partition	Solid / Glazed

Type

Framing		25mm deep x 125mm wide base channel

Junctions	40mm deep x 125mm base channel

25mm deep x 125mm abutment channel

40mm deep x 125mm abutment channel

Corners		125 x 125mm corner post

Panel to Panel joint with 3mm shadow-gap

Solid Panels

Type

15mm Laminated MDF

18mm MFC

18mm PET Panel (6mm PET +12mm MDF)

19mm Fabric Wrapped PET Panel

19mm Veneered MDF

Solid Panels

Finish and Colour  Insert Project Specific Finishes

Joint Treatment 		3mm Shadow-gap to panel joints

Type

Joint		Acoustic Seal

Treatment

Finish and Colour

Glazed Panels

Type

12.3mm magnetic writable glass

12.8mm acoustic laminate interlayer glass

Glazed Panels	Not Applicable

Integral Blinds

Glazed Panels 	N/A

Additional

Features

Door 		N/A

Arrangement

Type

Door 		N/A

Arrangement

Door Frames

Door 		N/A

Arrangement

Door Leafs

Type

Door		N/A

Arrangement

Door Leafs

Vision Panels

Additional	N/A

Apertures

Ceiling Type	Insert Project Specific Ceiling Type

Ceiling Panels	As Above

Ceiling Method	Timber fixing point required within the ceiling/bulkhead – to be supplied by others. Consult with ceiling manufacturer for support detail – co-ordination meeting with all parties

Of Support 	concerned to be arranged prior to internal glazing installation

Ceiling Joint	Secure Fixing into Timber provided above

Treatment

Perimeters	58mm Deep Outer Deflection Profile

Skirtings Sills

Trims		Extruded Aluminium Profiles

Hardware	Refer to RPG for more details

Seals		Black perimeter compression seals

Cable		Insert Project Specific Requirements

Management

And Outlets

Other		Refer to RPG for more Information

Features

Manifestation	To architect’s choice and in accordance with the requirements of Approved Document K:

Artwork and custom manifestation to client design Architect to confirm

Strength 	BS 5234:1992 non load-bearing screens not protecting a change in adjacent

Grade 		floor level of>380mm

For glass load resistance criteria, see below.

BS5234 not applied to “all glass partition screens”

Nominal line load criteria of “short-term impact load” utilised 0.36kN

Note: BS5234 where design load information is not supplied “all glass partition

screens, by default will support 0.36kN/m applied as linear load at 1100mm

above FFL”

Glass deflection limited to L/65 or 25mm, whichever is less under load

Maximum deflection and stress in accordance with the glass performance requirements of EN572/EN12150/EN12543/EN14179 (where heat soaked)

Building regulations Part N (BS6262 & BS EN 12600) + InsStucEng publication “structural use of glass in buildings”

NOTE: Architect to confirm if line load needs to be more than 0.36kN

GENERAL NOTE:

Relation to Smoke Seal to NFR systems

BS standards for smoke screens.BS 9999:2008 which supersedes BS5588-7 states the following:

In the absence of an appropriate method of test and performance

criteria, such construction should not contain unsealed joints and

permanently open or openable areas. Joints between such

construction and any abutting element should be tight and preferably

sealed with a filler conforming to BS EN 1366-3 or BS EN 1366-4

(e.g. plaster), a mastic, or a flexible strip (e.g. neoprene), as

appropriate.

As previously advised, RPG’s non-fire rated system can act as a smoke seal up to 150ºC, however if the smoke is likely to be hotter than 150ºC then one of our fire rated systems would need to be utilised.

K30	RELOCATABLE PARTITION SYSTEM

(Insert usage/location)

- Product Manufacturer:

Radii

31 Harbour Exchange Square

Canary Wharf London E14 9GE UK

T. 020 7952 4830

E. info@radiipartitioning.com

Product Reference:

- 2D Adaptable Walls

- Strength Grade:

For glass non loadbearing screens not protecting a change in floor level (380mm)

For glass load resistance criteria, see below;

- BS5234 n/a to “all” glass partition screens

- Nominal line load criteria of “accidental impact load” utilised

- 0.36kN/m applied as short term load at 1100mm above FFL

- Glass deflection limited to L/65 or 25mm, whichever is less under load

- Maximum deflection and stress in accordance with the glass performance requirements of EN572/EN12150/EN12543/EN14179(where heat soaked)

- Building regulations Part N(BS6262) + InsStucEng publication “structural use of glass in buildings”

BS 6180:2011 Barriers in and about buildings for Atria or protected change in adjacent floor levels

Horizontal uniformly distributed line load 1100mm above FFL (Lateral deflection limit L/65 or 25mm, whichever is less relative to glass fixings)

Structural Glass based on line load and attendant UDL + point load (delete as appropriate)

- Not Applicable

- 0.74kN/m

- 1.5kN/m

- 3.0kN/m

- Fire Resistance of complete system to BS 476-22:

- Not applicable.

- Sound Insulation to EN ISO 10140-2 and ISO 717-1: (delete as appropriate)

- 49dB Rw

- 50dB Rw

- 51dB Rw

- 52dB Rw

- 53dB Rw

- 54dB Rw

- 55dB Rw

- 56dB Rw

- 57dB Rw

- 58dB Rw

- 59dB Rw

- Partition Height:

- Raised Floor to Underside Ceiling - insert height mm

- Partition Abutments: (delete as appropriate)

- Head Detail:

Surface Mounted Deflection Head

No Deflection 	25mm deep x 125mm wide Aluminium Channel

±15mm Deflection	73mm deep x 125mm wide Two Part Aluminium Channel

±25mm Deflection 	83mm deep x 125mm wide Two Part Aluminium Channel

Recessed Deflection Head – Co-ordination required with ceiling contractor.

±15mm Deflection	15mm Visible Aluminium Channel

±25mm Deflection 	25mm Visible Aluminium Channel

- Floor Detail:

Surface mounted aluminium U channel screw fixed

25mm deep x 125mm wide as standard – larger base details available.

- Wall Abutments:

Surface mounted aluminium U channel screw fixed into drylined partition

25mm deep x 125mm wide as standard – larger abutment details available.

- Framing:

- Extruded Aluminium sections

- Glazing Junctions/Corners: (delete as appropriate)

- As design intent details.

- Glass to glass joints to be 2mm dry tape

- Skirting/Sills:

- Refer to floor detail options detailed above

- Trims:

- Not applicable – Refer to framing/head/floor/wall abutment clauses

- Solid Panels: (delete as appropriate)

- 12.3mm magnetic writable glass

- 12.8mm acoustic laminate interlayer glass

- 15mm Laminated MDF

- 18mm MFC

- 18mm PET Panel (6mm PET +12mm MDF)

- 19mm Fabric Wrapped PET Panel

- 19mm Veneered MDF

- Glazing Type:

- Not Applicable

- Manifestation:

- Not Applicable

- Blinds:

- Not Applicable

- Door Sets:

- Not Applicable

- Vision Panels:

- Not Applicable

- Ironmongery:

- Not Applicable

- Accessories:

- Refer to Radii for further information

- Special Requirement’s: (Delete as appropriate)

- Sustainability – System to conform to the requirements of

BREEAM

SKA

LEED

- BIM – System to come complete with Revit models

- Special Requirement’s (Insert Requirement’s)

- Finishes/Colour: (delete as appropriate)

- Polyester Powder Coated RAL (Insert Reference)

- Polished & Chemically brightened (Insert Reference)

- Linish & Chemically brightened (Insert Reference)

- Etch & Anodise (Insert Reference)

**Please note that this specification is

subject to design developments**